Optimizing Temporal Efficient Networks for Embedded Keyword Spotting on Microcontrollers

Abstract — Keyword spotting (KWS) enables voice-driven interaction on embedded devices. While recent lightweight architectures achieve high accuracy on standard benchmarks, most require millions of multiply-accumulate operations (MACCs), limiting deployment on the most resource-constrained microcontrollers. This work optimizes the Temporal Efficient Network (TENet) for keyword spotting under a strict computational budget matched to a DS-CNN embedded baseline. A four-stage progressive grid search over six architectural hyperparameters evaluates 2,302 configurations to identify an efficient TENet variant. The optimized model, trained on Google Speech Commands v2, achieves 96.84% test accuracy (96.76% after INT8 quantization) with 12,822 parameters and 276,136 MACCs. Deployed on an STM32F767IGT6 Cortex-M7 microcontroller, the INT8 model requires 13.87 KB flash and 7.64 KB RAM with 6.32 ms inference latency.

Keywords — Keyword spotting, TinyML, embedded AI, microcontroller, TENet, speech recognition, edge computing, neural architecture optimization.

# I. Introduction

Keyword spotting (KWS) is a speech processing task that detects predefined keywords from a continuous audio stream. It forms the foundation of voice-driven interfaces used in many modern applications such as smart speakers, wearable devices, and Internet-of-Things (IoT) systems [1]. In these applications, a lightweight KWS module continuously monitors incoming audio signals and activates higher-level processing only when a specific command or wake word is detected. This mechanism enables intuitive human–machine interaction while reducing unnecessary computation.

With the rapid growth of edge computing and IoT technologies, there is increasing demand for deploying speech recognition algorithms directly on embedded devices. Running KWS locally on edge hardware offers several advantages, including reduced latency, improved privacy, and lower communication overhead since audio data does not need to be transmitted to cloud servers [2]. However, implementing deep learning models on microcontrollers remains challenging due to strict constraints in memory capacity, computational power, and energy consumption [3].

To address these constraints, recent research has focused on developing lightweight neural network architectures for embedded keyword spotting. Zhang et al. [2] proposed depthwise separable CNN (DS-CNN) models specifically designed for microcontroller deployment, demonstrating real-time keyword spotting on ARM Cortex-M processors. Li et al. [4] introduced the Temporal Efficient Network (TENet), which uses multi-scale temporal convolutions to efficiently capture temporal patterns in speech signals. Kim et al. [5] proposed BC-ResNet with broadcasted residual connections, achieving high accuracy with compact architectures. Majumdar and Ginsburg [6] developed MatchboxNet using 1D time-channel separable convolutions. In these systems, compact acoustic features such as Mel-Frequency Cepstral Coefficients (MFCCs) [7] are commonly used to represent speech signals in a computationally efficient form. However, most reported high-accuracy KWS models require millions of MACCs, and their performance under extreme computational constraints (below 300K MACCs) remains largely unexplored.

Despite significant progress, designing efficient KWS systems for resource-constrained microcontrollers remains an active research problem. Most reported high-accuracy models require millions of multiply-accumulate operations (MACCs), making them impractical for the most constrained deployment scenarios. Selecting the most suitable neural architecture under strict computational limits is particularly challenging for TinyML applications [3], where both recognition accuracy and inference latency must be carefully balanced.

Motivated by these challenges, this work investigates the optimization of the TENet architecture [4] for keyword spotting on ARM Cortex-M7 microcontrollers. We adopt the computational budget of a DS-CNN embedded baseline [9] to enable direct comparison, and perform a progressive grid search over architectural hyperparameters to maximize accuracy within this constraint. The optimized model is trained on the Google Speech Commands v2 dataset [1] and deployed on an STM32F767IGT6 microcontroller using the ST X-CUBE-AI framework [8].

The remainder of this paper is organized as follows. Section II describes the proposed methodology. Section III presents the experimental results and comparison with existing approaches. Section IV concludes the paper.

The DS-CNN architecture proposed by Zhang et al. [2] established an influential baseline for keyword spotting on microcontrollers. Using depthwise separable convolutions, DS-CNN achieves competitive accuracy with significantly fewer parameters than standard CNNs. The model was deployed on an ARM Cortex-M7 processor with CMSIS-NN [16], demonstrating real-time inference within 70 KB of memory.

Li et al. [4] introduced TENet, which processes MFCC features as 1D temporal sequences using multi-scale temporal convolutions with an inverted bottleneck structure inspired by MobileNetV2 [10]. TENet achieves 96.8% accuracy on Google Speech Commands v2 with 100K parameters and 2.9M multiplications.

Kim et al. [5] proposed BC-ResNet, which uses broadcasted residual connections and sub-spectral normalization to achieve state-of-the-art accuracy (up to 98.7% with BC-ResNet-8) on the same benchmark. However, even the smallest variant (BC-ResNet-1, 9.2K parameters) requires 3.1M MACCs, which exceeds the computational budget considered in this work.

Majumdar and Ginsburg [6] developed MatchboxNet using 1D time-channel separable convolutions, achieving 97.48% accuracy with 93K parameters. Yang et al. [22] proposed LiCoNet, which uses linearized convolutions for efficient streaming inference on embedded platforms.

Neural architecture search (NAS) methods have been applied to optimize models for microcontroller deployment. Lin et al. [11] proposed MCUNet, which co-designs the neural architecture (TinyNAS) and inference engine (TinyEngine) to jointly optimize accuracy and latency under memory constraints. Differentiable NAS methods such as DARTS [12] and hardware-aware approaches such as MicroNAS [13] enable efficient exploration of large search spaces.

For smaller search spaces with a limited number of discrete hyperparameters, grid search and random search remain practical alternatives. Bergstra and Bengio [14] showed that random search can be more efficient than grid search in high-dimensional spaces, while successive halving strategies [15] reduce computational cost by progressively eliminating poor candidates. The progressive grid search approach used in this work combines exhaustive enumeration with staged elimination, which is effective for the six-dimensional search space considered.

Several frameworks support neural network deployment on microcontrollers. CMSIS-NN [16] provides optimized kernels for ARM Cortex-M processors. TensorFlow Lite for Microcontrollers (TFLM) [3] offers a portable runtime with INT8 quantization support. ST X-CUBE-AI [8] converts trained models (ONNX, TFLite, Keras) to optimized C code for STM32 microcontrollers, with support for INT8 quantized models in QDQ format.

The MLPerf Tiny benchmark [3] standardizes evaluation of ML models on microcontrollers, including a keyword spotting task with a DS-CNN reference model achieving ≥90% accuracy.

# II. Methodology

## A. System Overview

The proposed keyword spotting system follows a lightweight processing pipeline designed for deployment on resource-constrained microcontrollers. The system consists of three main stages: audio acquisition, feature extraction, and neural network inference.

Raw audio waveforms are captured at 16 kHz and segmented into one-second frames. The audio signal is then transformed into MFCC features, which provide a compact representation of the spectral characteristics of speech. These features are used as input to a lightweight neural network classifier that predicts the corresponding keyword class. The overall design minimizes computational complexity and memory usage to enable real-time inference on embedded hardware.

Fig. 1. System pipeline overview showing inference (top) and training/deployment (bottom) workflows.

## B. Dataset

All experiments in this work are conducted using the Google Speech Commands Dataset v2 [1], which is widely used for benchmarking keyword spotting systems.

The classification task consists of 12 classes, including ten keyword commands (yes, no, up, down, left, right, on, off, stop, go) together with two additional classes (unknown and silence). The unknown class groups together speech samples that do not correspond to the target commands. The silence class consists of zero-padded waveforms generated to comprise approximately 10% of the base training dataset size.

| Property | Value |
| --- | --- |
| Sampling rate | 16 kHz |
| Duration per sample | ~1 second (16,256 samples) |
| Training samples | ~51,000 |
| Validation samples | ~13,000 |
| Test samples | 12,105 |
| Number of classes | 12 |

The dataset uses the official train/validation/test split provided by the dataset authors.

## C. Feature Extraction

To convert raw audio signals into a representation suitable for neural network processing, Mel-Frequency Cepstral Coefficient (MFCC) features [7] are extracted from each audio sample using torchaudio.transforms.MFCC.

| Parameter | Value |
| --- | --- |
| Number of MFCC coefficients (n_mfcc) | 13 |
| Number of Mel filters (n_mels) | 16 |
| FFT size (n_fft) | 512 |
| Window length | 512 |
| Hop length | 256 |
| Minimum frequency (f_min) | 20 Hz |
| Center padding | False |
| Power | 2.0 |

These parameters produce an MFCC feature representation of 62 temporal frames × 13 coefficients per sample, resulting in an input tensor of shape (1, 62, 13) with a size of 3,224 bytes in float32 precision.

## D. TENet Architecture

TENet [4] is selected for its strong accuracy-efficiency trade-off: among lightweight 1D architectures, it achieves the highest accuracy at sub-300K MACCs while maintaining robust INT8 quantization behavior. TENet processes MFCC sequences using one-dimensional temporal convolutions. The input tensor (1, 62, 13) is reshaped to (13, 62), treating the 13 MFCC coefficients as input channels and the 62 frames as the temporal dimension.

The architecture adopts an inverted bottleneck design inspired by MobileNetV2 [10], consisting of pointwise expansion, depthwise temporal convolution, and pointwise projection with residual connections.

The network begins with a stem convolution (Conv1d, kernel=3), followed by multiple inverted bottleneck stages, and concludes with adaptive average pooling and a fully connected classification layer. The optimized architecture parameters obtained through grid search (Section II.E) are shown in Table II.

Table II: Optimized TENet architecture parameters.

| Parameter | Value |
| --- | --- |
| Stem channels | 14 |
| Block channels | [14, 14, 14, 14, 14] |
| Strides | [2, 2, 1, 1] |
| Expansion ratios | [4, 4, 4, 4] |
| Layers per block | [1, 2, 1, 1] |
| Kernel size | 9 |
| Dropout | 0.1 |
| Total parameters | 12,822 |
| Total MACCs | 276,136 |

Three key architectural design choices distinguish this configuration:

Fig. 2. TENet architecture: overall network structure (top) and inverted bottleneck block detail (bottom).

Narrow channels with high expansion ratio (14 ch × 4× expansion): This creates a rich intermediate representation (56 channels) for depthwise convolutions while maintaining a compact bottleneck. This is more efficient than wider channels with lower expansion (e.g., 24 ch × 2×) at the same MACC budget.

Double downsampling [2, 2, 1, 1]: Two stride-2 stages reduce the temporal dimension from 62 → 31 → 16 early in the network, allowing later blocks to operate on shorter sequences with 48% fewer temporal operations.

Extra depth at mid-resolution (layers=[1, 2, 1, 1]): Placing an additional layer at the 31→16 transition provides greater representational capacity where information density is highest, at a cost of only ~52K additional MACCs.

## E. Architecture Optimization

After selecting TENet as the base architecture, a four-stage progressive grid search was conducted to identify improved architectural configurations within the MACC budget of 287,673 (matching the DS-CNN baseline [9]).

The search explored six hyperparameters:

| Dimension | Values Explored | Winner |
| --- | --- | --- |
| Stem channels | {8, 12, 14, 16, 18, 20, 24} | 14 |
| Block channels | {14, 16, 18, 20, 22, 24, 28, 32} | 14 |
| Number of blocks | {2, 3, 4} | 4 |
| Expansion ratio | {1, 2, 3, 4} | 4 |
| Kernel size | {3, 5, 7, 9, 11, 13, 15} | 9 |
| Layers per block | Various combinations | [1, 2, 1, 1] |

Stride patterns were determined by the number of blocks: three candidate patterns per block count (e.g., [2,1,1,1], [2,2,1,1], [2,1,1,2] for 4 blocks). Any candidate exceeding the MACC budget was discarded before training.

The search was conducted in four progressive stages, summarized in Table III:

Table III: Four-stage progressive grid search strategy.

| Stage | Candidates | Epochs | Augmentation | GPU Hours | Selection |
| --- | --- | --- | --- | --- | --- |
| Round 1 (Coarse) | 1,280 | 15 | None | 2.7 | Top val accuracy |
| Round 2a (Fine) | 548 | 15 | None | 1.1 | Expanded grid |
| Round 2b (Deep) | 464 | 200 | All (§II.F) | 12.0 | Val accuracy |
| Round 2c (Final) | 10 | 1,000 | All | 0.5 | Final ranking |
| Total | 2,302 | — | — | ~16 | — |

All experiments were conducted on an NVIDIA RTX 5080 Laptop GPU with data preloaded to GPU VRAM for efficient small-model training. The progressive search strategy is conceptually related to successive halving [15], where computational resources are concentrated on the most promising candidates in later stages.

Fig. 3. Progressive grid search: accuracy vs. computational cost across four rounds (2,302 candidates).

## F. Training Configuration

The best architecture identified by grid search (Section II.E) was trained to convergence on an NVIDIA RTX 5080 Laptop GPU using the following configuration:

| Parameter | Value |
| --- | --- |
| Optimizer | AdamW |
| Learning rate | 0.001 |
| Weight decay | 1×10⁻⁴ |
| Batch size | 1,024 |
| Maximum epochs | 300 |
| Best epoch | 273 |
| Early stopping patience | 50 epochs |
| LR scheduler | CosineAnnealingLR + 5-epoch linear warmup |
| Minimum LR (η_min) | 1×10⁻⁵ |
| Random seed | 123 |

Four data augmentation and regularization techniques were applied:

SpecAugment [17]: 2 time masks (width=5 frames) and 1 frequency mask (width=2 bins) applied to MFCC features during training.

Mixup [18]: Convex combination of training pairs with α=0.2, applied at the input level.

Label smoothing [19]: Cross-entropy loss with smoothing factor ε=0.1.

Cosine annealing [20]: Learning rate decayed from 0.001 to 10⁻⁵ following a cosine schedule after a 5-epoch linear warmup.

These techniques collectively contributed a +1.84 percentage point improvement in validation accuracy compared with training without augmentation (see Section III.C for ablation results).

Fig. 4. Training curve showing validation accuracy and cosine annealing learning rate schedule over 273 epochs.

## G. INT8 Quantization

Post-training static quantization was applied to convert the trained float32 model to INT8 precision for efficient embedded inference.

| Parameter | Value |
| --- | --- |
| Tool | ONNX Runtime quantize_static |
| Quantization format | QDQ (QuantizeLinear/DequantizeLinear) |
| Activation type | QInt8 (signed 8-bit, per-tensor) |
| Weight type | QInt8 (signed 8-bit, per-channel) |
| Activation symmetric | False (asymmetric) |
| Weight symmetric | True |
| Calibration set | 200 random training samples (seed=42) |
| ONNX opset | 13 |

The QDQ format inserts explicit quantization and dequantization nodes in the ONNX graph, which is directly supported by the ST X-CUBE-AI conversion tool [8]. Per-channel weight quantization preserves accuracy better than per-tensor quantization for depthwise convolution layers.

## H. Embedded Deployment

The quantized INT8 ONNX model was deployed on the STM32F767IGT6 microcontroller using ST X-CUBE-AI v2.2.0 [8].

| Property | Value |
| --- | --- |
| MCU | STM32F767IGT6 |
| Core | ARM Cortex-M7 |
| Clock frequency | 216 MHz |
| Flash memory | 1 MB |
| SRAM | 512 KB |
| FPU | Single-precision |
| AI framework | X-CUBE-AI v2.2.0 |

The deployment pipeline consists of four steps: (1) training in PyTorch (float32); (2) export to ONNX format (opset 13); (3) INT8 quantization using ONNX Runtime (QDQ format); and (4) code generation using X-CUBE-AI, producing optimized C library and weight arrays. Inference latency is measured using the ARM DWT (Data Watchpoint and Trace) cycle counter, which provides cycle-accurate timing of the ai_run() function call. The latency measurement covers only the neural network forward pass (ai_run); MFCC feature extraction is excluded as it is performed by the host application prior to inference.

# III. Results and Discussion

## A. Classification Performance

Table IV presents the overall classification performance of the optimized TENet model in both float32 and INT8 precision.

Table IV: Overall classification performance.

| Metric | Float32 | INT8 | Δ |
| --- | --- | --- | --- |
| Test Accuracy | 96.84% | 96.76% | −0.08% |
| Weighted F1 | 0.9683 | 0.9675 | −0.0008 |
| Macro F1 | 0.9504 | 0.9490 | −0.0014 |
| Macro Precision | 0.9604 | 0.9573 | −0.0031 |
| Macro Recall | 0.9410 | 0.9411 | +0.0001 |

INT8 quantization causes only minimal accuracy degradation (−0.08%), demonstrating that the quantized model preserves the predictive capability of the floating-point model while enabling efficient embedded deployment. Per-class F1-scores range from 0.918 (go) to 1.000 (silence), with INT8 degradation below 0.011 for all classes. The macro F1-score (0.9490) provides a balanced assessment across the 12 classes, as the unknown class (57.3% of test samples) dominates the weighted metric.

## B. Ablation Study

To quantify the contribution of training techniques, Table VI presents an ablation study comparing different training configurations.

Table VI: Ablation study of architecture optimization and training techniques.

| Configuration | Val Acc | Test Acc | Δ vs. Previous |
| --- | --- | --- | --- |
| Baseline TENet (manual arch, no aug.) | 95.56% | 95.13% | — |
| Grid-optimized arch, no aug., 15 ep | 95.56% | — | +0.00% |
| Grid-optimized arch + all aug., 200 ep | 97.09% | — | +1.53% |
| Grid-optimized arch + all aug., 300 ep | 97.40% | 96.84% | +0.31% |

The results show that architecture optimization alone provides a modest improvement when evaluated at short training horizons (15 epochs). Training augmentation techniques (SpecAugment, Mixup, label smoothing, cosine annealing) contribute +1.84% validation accuracy improvement, demonstrating that training methodology is as important as architecture design for small models. Extended training (300 epochs with early stopping at epoch 273) provides an additional +0.31% improvement.

The gap between validation accuracy (97.40%) and test accuracy (96.84%) is 0.56%, which is within the expected range for a single-seed evaluation and reflects the selection bias inherent in choosing the best architecture from 2,302 candidates.

## C. Model Size and Deployment Metrics

Table VII: Model size and deployment metrics (INT8).

| Metric | Value |
| --- | --- |
| Parameters | 12,822 |
| MACCs (model) | 276,136 |
| MACCs (X-CUBE-AI, incl. QDQ overhead) | 280,389 |
| Flash memory (weights) | 13.87 KB |
| RAM (activations) | 7.64 KB |
| Firmware binary size | 67.4 KB |
| Weight compression vs. float32 | −70.8% |

The MACC count reported by X-CUBE-AI (280,389) is slightly higher than the model-intrinsic count (276,136) due to additional operations introduced by QuantizeLinear and DequantizeLinear nodes in the INT8 inference graph.

## D. On-Device Inference Latency

Inference latency was measured directly on the STM32F767IGT6 microcontroller using the DWT cycle counter.

Table VIII: On-device inference latency comparison.

| Model | MCU | Clock | Latency | MACCs | Cycles/MACC |
| --- | --- | --- | --- | --- | --- |
| Ours (TENet INT8) | STM32F767 | 216 MHz | 6.32 ms | 276K | ~4.9 |
| DS-CNN baseline [9] | STM32F769NI | 216 MHz | 7.00 ms | 288K | ~5.3 |

The optimized TENet model achieves 6.32 ms inference latency, which is 9.7% faster than the DS-CNN baseline despite operating on the same Cortex-M7 core at the same clock frequency. The ~4.9 cycles per MACC is consistent with INT8 mixed-precision inference on the Cortex-M7 architecture using X-CUBE-AI optimized kernels.

## E. Comparison with State-of-the-Art

Table IX compares the proposed model with previously reported keyword spotting architectures. Two categories of comparison are presented: (1) models evaluated on GSCv2 12-class under similar MACC constraints, and (2) models deployed on microcontrollers with reported on-device metrics.

Table IX: Comparison with state-of-the-art keyword spotting models.

| Model | Year | Params | MACCs | Accuracy | Flash | RAM | Latency | Platform |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ours (INT8) | 2026 | 12.8K | 276K | 96.76% | 13.87 KB | 7.64 KB | 6.32 ms | STM32F767 (M7@216MHz) |
| DS-CNN [9] | 2020 | ~6K | 288K | 91.00% | 169.63 KB | 11.52 KB | 7.00 ms | STM32F769NI (M7@216MHz) |
| DS-CNN-S [2] | 2018 | ~23K | 5.4M | 94.4%† | 38.6 KB | ~70 KB | ~12 ms | STM32F746 (M7@216MHz) |
| TENet12 [4] (12-class) | 2020 | 100K | 2.9M | 96.8% | — | — | — | — |
| TENet6-N [4] (12-class) | 2020 | 17K | 553K | 96.0% | — | — | — | — |
| BC-ResNet-1 [5] (12-class) | 2021 | 9.2K | 3.1M | 96.9% | — | — | — | — |
| MatchboxNet [6] (12-class) | 2020 | 93K | — | 97.48% | — | — | — | — |
| MLPerf Tiny [3] | 2021 | 38.6K | ~100K | ≥90% | — | ≤512 KB | ~182 ms | NUCLEO-L4R5ZI (M4@120MHz) |

†Zhang et al. [2] evaluated on GSCv1, not GSCv2; results are not directly comparable.

Note: Bartoli et al. [21] recently evaluated several models (DS-CNN, TENet6, LiCoNet-S, TKWS) on STM32 platforms, reporting F1-scores of 91.2%–93.6%. However, their evaluation used an 8-class task with different MFCC configurations (15×63), making direct numerical comparison with our 12-class results inappropriate. Their study does confirm that TENet and LiCoNet architectures are more energy-efficient than DS-CNN on Cortex-M7 processors.

Compared with the DS-CNN baseline [9] evaluated on the same MCU architecture (Cortex-M7 at 216 MHz) under the same MACC constraint (~287K):

| Metric | DS-CNN Baseline [9] | Ours (TENet INT8) | Improvement |
| --- | --- | --- | --- |
| Accuracy | 91.00% | 96.76% | +5.76 pp |
| Flash (weights) | 169.63 KB | 13.87 KB | −91.8% |
| RAM (activations) | 11.52 KB | 7.64 KB | −33.7% |
| Inference latency | 7.00 ms | 6.32 ms | −9.7% |

Compared with the original TENet paper [4], our optimized variant achieves comparable accuracy (96.76% vs. 96.8% for TENet12+MTConv) while requiring approximately 10× fewer MACCs (276K vs. 2.9M). This demonstrates that systematic architecture optimization can recover most of the accuracy of larger models within a severely constrained computational budget.

# IV. Conclusion

This paper presented an optimized keyword spotting system for ARM Cortex-M7 microcontrollers. A four-stage progressive grid search over 2,302 TENet configurations was conducted under a computational budget matched to a DS-CNN embedded baseline. The best configuration was trained with SpecAugment, Mixup, label smoothing, and cosine annealing, then quantized to INT8 and deployed on an STM32F767IGT6 using X-CUBE-AI.

The optimized model achieves 96.76% INT8 accuracy (weighted F1 = 0.9675) on the Google Speech Commands v2 12-class benchmark with 12,822 parameters and 276K MACCs. On-device, the INT8 model requires 13.87 KB flash, 7.64 KB RAM, and 6.32 ms inference latency at 216 MHz.

This work has several limitations. The evaluation is based on a single training seed, and the reported accuracy may vary by ±0.2–0.5% across different random initializations. Energy consumption was not measured, which is an important metric for battery-powered devices. The grid search approach, while effective for the six-dimensional space considered, does not scale to larger or continuous search spaces.

Future work will explore: (1) multi-seed evaluation to establish confidence intervals; (2) energy efficiency analysis using current measurement on the target hardware; (3) extension to streaming/continuous keyword spotting scenarios; (4) hardware-aware NAS methods (e.g., differentiable NAS, Bayesian optimization) to explore larger architecture spaces; and (5) deployment on additional MCU platforms including Cortex-M4 and RISC-V processors.

# References

[1] P. Warden, "Speech commands: A dataset for limited-vocabulary speech recognition," arXiv preprint arXiv:1804.03209, 2018.

[2] Y. Zhang, N. Suda, L. Lai, and V. Chandra, "Hello edge: Keyword spotting on microcontrollers," arXiv preprint arXiv:1711.07128, 2017.

[3] C. Banbury et al., "MLPerf Tiny benchmark," in Proc. Neural Information Processing Systems (NeurIPS) Datasets and Benchmarks Track, 2021.

[4] X. Li, X. Wei, and X. Qin, "Small-footprint keyword spotting with multi-scale temporal convolution," in Proc. Interspeech, 2020, pp. 1987–1991.

[5] B. Kim, S. Chang, J. Lee, and D. Sung, "Broadcasted residual learning for efficient keyword spotting," in Proc. Interspeech, 2021.

[6] S. Majumdar and B. Ginsburg, "MatchboxNet: 1D time-channel separable convolutional neural network architecture for speech commands recognition," in Proc. Interspeech, 2020.

[7] S. Davis and P. Mermelstein, "Comparison of parametric representations for monosyllabic word recognition in continuously spoken sentences," IEEE Trans. Acoustics, Speech, and Signal Processing, vol. 28, no. 4, pp. 357–366, 1980.

[8] STMicroelectronics, "X-CUBE-AI: AI expansion pack for STM32CubeMX," 2024.

[9] P. M. Sorensen, B. Epp, and T. May, "A depthwise separable convolutional neural network for keyword spotting on an embedded system," EURASIP J. Audio, Speech, and Music Processing, vol. 2020, no. 1, pp. 1–11, 2020.

[10] M. Sandler, A. Howard, M. Zhu, A. Zhmoginov, and L.-C. Chen, "MobileNetV2: Inverted residuals and linear bottlenecks," in Proc. IEEE/CVF CVPR, 2018, pp. 4510–4520.

[11] J. Lin, W.-M. Chen, Y. Lin, J. Cohn, C. Gan, and S. Han, "MCUNet: Tiny deep learning on IoT devices," in Proc. NeurIPS, 2020.

[12] H. Liu, K. Simonyan, and Y. Yang, "DARTS: Differentiable architecture search," in Proc. ICLR, 2019.

[13] Y. Qiao, H. Xu, Y. Zhang, and S. Huang, "MicroNAS: Zero-shot neural architecture search for MCUs," in Proc. DATE, 2024.

[14] J. Bergstra and Y. Bengio, "Random search for hyper-parameter optimization," J. Machine Learning Research, vol. 13, pp. 281–305, 2012.

[15] K. Jamieson and A. Talwalkar, "Non-stochastic best arm identification and hyperparameter optimization," in Proc. AISTATS, 2016.

[16] L. Lai, N. Suda, and V. Chandra, "CMSIS-NN: Efficient neural network kernels for Arm Cortex-M CPUs," arXiv preprint arXiv:1801.06601, 2018.

[17] D. S. Park et al., "SpecAugment: A simple data augmentation method for automatic speech recognition," in Proc. Interspeech, 2019.

[18] H. Zhang, M. Cissé, Y. N. Dauphin, and D. Lopez-Paz, "Mixup: Beyond empirical risk minimization," in Proc. ICLR, 2018.

[19] C. Szegedy, V. Vanhoucke, S. Ioffe, J. Shlens, and Z. Wojna, "Rethinking the inception architecture for computer vision," in Proc. IEEE/CVF CVPR, 2016.

[20] I. Loshchilov and F. Hutter, "SGDR: Stochastic gradient descent with warm restarts," in Proc. ICLR, 2017.

[21] P. Bartoli, T. Bondini, C. Veronesi, A. Giudici, N. Antonello, and F. Zappa, "End-to-end efficiency in keyword spotting: A system-level approach for embedded microcontrollers," arXiv preprint arXiv:2509.07051, 2025.

[22] H. Yang et al., "LiCo-Net: Linearized convolution network for hardware-efficient keyword spotting," arXiv preprint arXiv:2211.04635, 2022.